OSCN LAB – 4

Name: NAMILLA SIRI

HALL TICKET NO.: 2503B05137

PROGRAM 4 – To implement Error Detection using Cyclic Redundancy Check.

TASK 1 : A bit stream 1101011011 is transmitted using the standard CRC method. The generator polynomial is x 4 +x+1. What is the actual bit string Transmitted.

TASK 2 : A bit stream 10011101 is transmitted using the standard CRC method. The generator polynomial is x 3 +1. What is the actual bit string transmitted. Suppose the third bit from the left is inverted during transmission. How will the receiver detect this error.

__________________________________________________________________

TASK 1 CODE:

#include <iostream>

#include <string>

using namespace std;

// Function to perform XOR on two bits

char xor_bit(char a, char b) {

return (a == b) ? '0' : '1';

}

// Function to perform Modulo-2 Division

string mod2_division(string dividend, string divisor) {

int pick = divisor.length();

string temp = dividend.substr(0, pick);

while (pick < dividend.length()) {

if (temp[0] == '1') {

// XOR with divisor

for (int i = 1; i < divisor.length(); i++)

temp[i] = xor_bit(temp[i], divisor[i]);

} else {

// XOR with all 0's

for (int i = 1; i < divisor.length(); i++)

temp[i] = xor_bit(temp[i], '0');

}

temp.erase(temp.begin());

temp += dividend[pick];

pick++;

}

// Last step

if (temp[0] == '1') {

for (int i = 1; i < divisor.length(); i++)

temp[i] = xor_bit(temp[i], divisor[i]);

} else {

for (int i = 1; i < divisor.length(); i++)

temp[i] = xor_bit(temp[i], '0');

}

temp.erase(temp.begin());

return temp;  // Remainder

}

int main() {

string data, generator;

cout << "Enter the data bitstream: ";

cin >> data;

cout << "Enter generator polynomial in binary (example x^4 + x + 1 = 10011): ";

cin >> generator;

int r = generator.length() - 1;

// Append r zeros to data

string data_appended = data + string(r, '0');

// Perform modulo-2 division

string remainder = mod2_division(data_appended, generator);

// Final transmitted frame

string transmitted = data + remainder;

cout << "\nCRC Remainder: " << remainder << endl;

cout << "Transmitted Bit String: " << transmitted << endl;

return 0;

}

Output:

Enter the data bitstream: 1101011011

Enter generator polynomial in binary: 10011

CRC Remainder: 1110

Transmitted Bit String: 11010110111110

TASK – 2 CODE

#include <iostream>

#include <string>

using namespace std;

char xor_bit(char a, char b) {

return (a == b) ? '0' : '1';

}

string mod2_division(string dividend, string divisor) {

int pick = divisor.length();

string temp = dividend.substr(0, pick);

while (pick < dividend.length()) {

if (temp[0] == '1') {

for (int i = 1; i < divisor.length(); i++)

temp[i] = xor_bit(temp[i], divisor[i]);

} else {

for (int i = 1; i < divisor.length(); i++)

temp[i] = xor_bit(temp[i], '0');

}

temp.erase(temp.begin());

temp += dividend[pick];

pick++;

}

if (temp[0] == '1') {

for (int i = 1; i < divisor.length(); i++)

temp[i] = xor_bit(temp[i], divisor[i]);

} else {

for (int i = 1; i < divisor.length(); i++)

temp[i] = xor_bit(temp[i], '0');

}

temp.erase(temp.begin());

return temp;  // Remainder

}

int main() {

string data, generator;

cout << "Enter the data bitstream: ";

cin >> data;

cout << "Enter generator polynomial in binary (example x^3 + 1 = 1001): ";

cin >> generator;

int r = generator.length() - 1;

// Append zeros

string appended = data + string(r, '0');

// CRC remainder

string remainder = mod2_division(appended, generator);

// Transmitted codeword

string transmitted = data + remainder;

cout << "\nCRC Remainder: " << remainder << endl;

cout << "Actual transmitted bit string: " << transmitted << endl;

// Simulate error: invert 3rd bit

string received = transmitted;

if (received.length() >= 3)

received[2] = (received[2] == '0') ? '1' : '0';

cout << "\nSimulated received bit string (3rd bit inverted): " << received << endl;

// Receiver performs CRC check

string receiver_remainder = mod2_division(received, generator);

if (receiver_remainder.find('1') != string::npos) {

cout << "\nReceiver Remainder: " << receiver_remainder << endl;

cout << "ERROR DETECTED: Received frame is corrupted.\n";

} else {

cout << "\nReceiver Remainder: " << receiver_remainder << endl;

cout << "NO ERROR DETECTED: Frame is correct.\n";

}

return 0;

}

Output:

Enter the data bitstream: 10011101

Enter generator polynomial in binary (example x^3 + 1 = 1001): 1001

CRC Remainder: 101

Actual transmitted bit string: 10011101101

Simulated received bit string (3rd bit inverted): 10111101101

Receiver Remainder: 110

ERROR DETECTED: Received frame is corrupted.